MSDS Number: G-1131B* *  * * Effective Date: 07/02/07 * * * * * Supercedes: 02/16/06

# Material Safety Data Sheet

# Cascoset G-1131B

# Paraformaldehyde Catalyst/Hardener

## 1. Product Identification

Description:	Cascoset G-1131B powdered catalyst

Synonyms: 	Paraformaldehyde hardener

## 2. Hazardous Ingredients

Ingredient,                             		CAS No         		Percent

Hazardous                                  					By Weight

---------------------------------------  		 ------------  		 ----------

Formaldehyde	        	             		50-00-0         		0.1-1.0%

Paraformaldehyde                         		30525-89-4     		50-70%

## 3. Physical Data

## Appearance & Odor:	Tan/light brown powder, distinct formaldehyde odor

## Storage Life:		1-2 years

## pH:			3-5

## Boiling Point:		not applicable

## Flash Point:		160°F  PMCC

## Freezing Point:		not applicable

## Solubility in Water:	completely dispersible

## Vapor Density:		>1

## Vapor Pressure:		1 mm Hg

## 4. Health Hazards

Acute Hazard:

Ingestion:

Not expected to be harmful under normal conditions of use.  If accidentally swallowed, burns or

irritation to mucous  membranes, esophagus or GI tract can result.

Inhalation:

May be harmful if inhaled.  May cause irritation of nose, throat and lungs.

Skin Contact:

May cause irritation with prolonged or repeated contact.

Eye Contact:

Causes chemical burns.

Chronic Hazard:

Formaldehyde:

Probable human carcinogen.  May cause allergic skin reaction.  May cause respiratory

sensitization.  Pre-existing respiratory and skin disorders may be aggravated by exposure.

Paraformaldehyde:

May cause allergic skin reaction.

SAF-T-DATA(tm) HMIS Ratings 
----------------------------------------------------------------------------------------------------------- 
Health Rating: 3 - Severe 
Flammability Rating: 2 - Moderate 
Reactivity Rating: 1 - Slight 
Contact Rating:  X – n/a 
Lab Protective Equip: GOGGLES; LAB COAT; VENT HOOD; PROPER GLOVES 
 -----------------------------------------------------------------------------------------------------------

## 5. First Aid Measures

Inhalation: 
Remove to fresh air. If not breathing, give artificial respiration. If breathing is difficult, give oxygen. Call a physician. 
Ingestion: 
If accidentally swallowed, DO NOT INDUCE VOMITING. Give large quantities of water. Never give anything by mouth to an unconscious person. Get medical attention immediately. 
Skin Contact: 
In case of contact and irritation, flush with water.    
Eye Contact: 
Immediately flush eyes with plenty of water for at least 15 minutes, lifting lower and upper eyelids occasionally.  Get medical attention.

6. Fire & Explosion Hazards:

Combustible.  Keep away from heat, sparks, flame and other ignition sources.

In case of fire, use water spray, dry chemical, “alcohol” foam or CO2.  Use water to keep fire-exposed containers cool.

7. Accidental Release Measures

Eliminate sources of ignition.  Scoop or sweep up and remove to a chemical disposal container.  Prevent entry into natural bodies of water.  Do not flush to sewer!  Dispose of according to local, state, and federal requirements.

8. Storage

Keep in a tightly closed container to prevent contact with moisture in air.  Store in a cool, dry, ventilated area.  Protect against physical damage.  Isolate from any source of heat or ignition.

9. Safe Handling & Personal Protection

Personal Protection:

Where air contaminants can exceed acceptable criteria, use NIOSH/MSHA approved respiratory protection equipment.  Respirators should be selected based on the form and concentration of contaminants in air in accordance with OSHA 29 CFR 1910-134 or other applicable standards or guidelines.  If airborne contaminants are generated when the material is heated or handled, sufficient ventilation in volume and airflow patterns should be provided to keep air contaminant concentration levels below acceptable criteria.  Use goggles if contact is likely.  Wear impervious gloves to prevent skin contact.

Safe Handling:

Inhalation:  	Avoid breathing dust.  Use adequate ventilation.

Skin Contact:	Avoid prolonged or repeated contact with skin  and clothing.

Eye Contact:	Do not get in eyes.

Handle in accordance with good industrial hygiene and safety practices.  These practices include avoiding unnecessary exposure and removal of the material from eyes, skin, and clothing.  Wash hands thoroughly after handling.

10. Stability and Reactivity

Conditions to Avoid: 
Normally stable, but may become unstable at high temperatures or may react with water.

Incompatibilities: 
Reacts with many compounds.  Reaction with phenol, acids, and alkalis may be violent.

Reaction with hydrochloric acid may form a bis-chloromethyl ether, an OSHA regulated carcinogen.  Incompatible with strong bases and oxidizing agents.

Hazardous Decomposition Products: 
Normal decomposition products include CO2, NOx, hydrogen cyanide, aldehydes including formaldehyde, particulate matter and other organic compounds.

Stability: 
Will polymerize wt high temperatures with some evolution of heat.  Stable under recommended storage conditions.

Hazardous Polymerization: 
This product should be treated as a combustible dust in the finely divided and suspended state.  Hazardous polymerization can occur at high temperatures.  Hazardous polymerization will not occur under normal conditions.

## 11. Toxicological Information

--------\Cancer Lists\-----------------------------------------------

---NTP Carcinogen---

Ingredient                                            Known    Anticipated    IARC

------------------------------------               -----         -----------       -----

Formaldehyde (50-00-0)                        No              Yes            Yes

Paraformaldehyde (30525-89-4)            No              No              No

Significant Data with Possible Relevance to Humans:

This product contains trace amounts of free formaldehyde.  OSHA and NTP identify formaldehyde as a potential carcinogen.  IARC identifies formaldehyde as a human carcinogen.  Formaldehyde has been shown to cause mutations in a variety of in-vitro test systems, the significance of which to humans is currently unknown.  In a two-year inhalation study, rats showed carcinogenic effects in the respiratory system at 15 ppm of formaldehyde.  There should be minimal risk when used with ventilation adequate to keep the atmospheric concentration of formaldehyde below the recommended exposure limits.  Maintain adequate ventilation to prevent exposure above current OSHA/ACGIH exposure limits.  Workplace monitoring of the air to define formaldehyde exposure levels may be necessary.

## 12. Ecological Information

Environmental Fate: 
Ecological injuries are not known or expected under normal use.

13. Disposal Considerations

Dispose of material in accordance with all federal, state, and local regulations.  State and local regulations/restrictions are complex and may differ from federal regulations.  Responsibility for proper waste disposal is with the owner of the waste.

EPA Waste Code if Discarded (40 CFR Section 261):  This product does not meet the definition of a hazardous waste according to U.S. EPA Hazardous Waste Management Regulation, 40 CFR Section 261.

14. Transport Information

DOT Proper Shipping Name: 	Consumer Commodity, ORM-D

Domestic Air Shipping name:	“Paraformaldehyde Mixture”

Hazard Class:  4.1 for air shipments

Packing Group:   III

UN #2213

## 15. Regulatory Information

CERCLA—SARA Hazard Category:

This product has been reviewed according to the EPA “Hazard Categories” promulgated under Sections 311 and 312 of the Superfund Amendment and Reauthroization Act of 1986 (SARA Title III) and is considered, under applicable definitions, to meet the following categories:

- Immediate Health Hazard, Chronic Health Hazard, Fire Hazard, Reactive Hazard

SARA Section 313:

This product contains the following substances subject to the reporting requirements of Section 313 of Title III of the Superfund Amendment and Reauthorization Act of 1986 and 40 CFR part 372:

- Formaldehyde

Toxic Substances Control Act:

All ingredients in this product are either on TSCA inventory list, or otherwise exempt.

This product contains the following chemical substances subject to the reporting requirements of TSCA 12(B) if exported from the United States:

None