Material Safety Data Sheet

EPOCAST 169 A, FPC2114 -- 8040-00N041931

=====================  Product Identification  =====================

Product ID:	EPOCAST 169 A, FPC2114

MSDS Date:	01/25/1990

FSC:		8040

NIIN:		00N041931

MSDS Number: 	BSVBW

=== Manufacturer ===

Company Name:		Huntsman

Address:			5121 SAN FERNANDO RD WEST

LOS ANGELES

Info Phone Num:		818-247-6210

Emergency Phone Num:	818-247-6210; 800-888-8372

=============  Composition/Information on Ingredients  =============

Ingredient Name:	QUATERNARY AMMONIUM COMPOUNDS,

BIS(HYDROGENATED TALLOW ALKYL) DIMETHYL-SALT

W/BENTONITE; (BENTONITE CLAY 27) (ING 7)

CAS:		68953-58-2

RTECS #:	UZ2996400

Fraction by Wt: 	5%

Ingredient Name:	ING 6:(ORGANOPHILIC CLAY)

RTECS #:	9999999ZZ

Ingredient Name:	PHENOL, 4,4'-ISOPROPYLIDENEDI-, POLYMER

W/1-CHLORO-2,3-EPOXYPROPANE; (DGBA EPOXY RESINS) (DIGLYCIDYLETHER OF (ING 2)

CAS:		25068-38-6

RTECS #:	SL6475000

Fraction by Wt: 	60%

Ingredient Name:	ING 1:BISPHENOL A).

LD50:		(ORAL,RAT) >5,000 MG/KG.

RTECS #:	9999999ZZ

Ingredient Name:	PHTHALIC ACID, DIBUTYL ESTER; (1,2-BENZENEDICARBOXYLIC

ACID, DIBUTYL ESTER)  (SARA III)

CAS:		84-74-2

RTECS #:	TI0875000

Fraction by Wt: 	15%

OSHA PEL:	5 MG/M3

ACGIH TLV:	5 MG/M3

EPA Rpt Qty:	10 LBS

DOT Rpt Qty:	10 LBS

Ingredient Name:	SILICA, AMORPHOUS FUMED; (SILICA, AMORPHOUS)

CAS:		112945-52-5

RTECS #:	VV7310000

Fraction by Wt: 	5%

OSHA PEL:	6 MG/M3 TDUST

ACGIH TLV:	10 MG/M3 TDUST

Ingredient Name:	PHENOL FORMALDEHYDE RESIN; (PHENOL, POLYMER WITH

FORMALDEHYDE)

CAS:		9003-35-4

Fraction by Wt: 	15%

=====================  Hazards Identification  =====================

LD50 LC50 Mixture:	SEE INGREDIENT 1.

Routes of Entry: 		Inhalation:YES  Skin:YES  Ingestion:NO

Reports of Carcinogenicity:	NTP:NO    IARC:NO	OSHA:NO

Health Hazards Acute and Chronic:	ACUTE:INHAL:VAP/FUMES CAN CAUSE IRRIT

TO THROAT & NOSE. EYE:SUBSTANCES CAN CAUSE SUBSTANTIAL IRRIT.

SKIN:SUBSTANCE CAN CAUSE MOD IRRIT. SUBSTANCE CAN BE ABSORBED THRU

INTACT SKIN IN HARMFUL AMTS. INGE ST:SUBSTANCE CAN CAUSE STUPOR,

DIZZ, FEVER, DROW, NAUS, VOMIT & CAUSE DMG TO KIDNEY.

CHRONIC:PRLNG/RPTD(EFTS OF OVEREXP)

Explanation of Carcinogenicity:	NOT RELEVANT

Effects of Overexposure:	HLTH HAZ:EXPOS CAN CAUSE LIVER & KIDNEY DMG &

BLOOD CHANGES. MAY CAUSE BIRTH DEFECTS & REPRO DISORDERS. ALLERGIC

DERM/RESP RESPONSE IN SUSCEPTIBLE INDIVIDUALS MAY BE DELAYED. IT

MAY APPEAR AFTER WEEKS  OR EVEN MONTHS OF FREQUENT & PRLNG CONTACT.

Medical Cond Aggravated by Exposure:	ALLERGY, ECZEMA, EYE OR RESPIRATORY

CONDITIONS.

=======================  First Aid Measures  =======================

First Aid:

Inhalation:  Move subject to fresh air.

Eyes:  Flush w/large amount of water for at least 15 minutes.  Get prompt medical attention.

Skin Contact:  Wash thoroughly w/soap & water.  Remove and wash clothing before re-use.

Ingestion:  Dilute by giving 2 glasses of water to drink.  Get medical attention.

Note:  Never give anything by mouth to unconscious person.

=====================  Fire Fighting Measures  =====================

Flash Point Method:		PMCC

Flash Point:			489F,254C

Extinguishing Media:		CO2, DRY CHEMICAL, WATER SPRAY, FOAM.

Fire Fighting Procedures:		WEAR NIOSH/MSHA APPROVED PRESSURE DEMAND SCBA

AND PROTECTIVE EQUIPMENT .

Unusual Fire/Explosion Hazard:	DECOMPOSITION & COMBUSTION PRODUCTS MAY

BE TOXIC.

==================  Accidental Release Measures  ==================

Spill Release Procedures:	CLEAN UP ALL TRACES OF SPILL. SHOVEL AS MUCH

OF MATERIAL AS POSSIBLE INTO DRY, CLEAN CONTAINERS. ON HARD SURFACE USE ABSORBENT MATERIAL TO PICK UP REMAINDER; ON LOOSE SURFACES SHOVEL UP CONTAMINATED

LAYER.

Neutralizing Agent:	NONE SPECIFIED BY MANUFACTURER.

======================	Handling and Storage  ======================

Handling and Storage Precautions:MAX STOR TEMP:35C. STORE AWAY FROM

EXCESS HEAT (E.G., STEAMPIPES, RADIATORS), FROM SOURCES OF IGNIT &

FROM REACTIVE MATLS.

Other Precautions:AVOID ALL SKIN CONTACT, MAY CAUSE ALLERGIC REACTION.

PLEASE READ MSDS BEFORE HANDLING THE PRODUCT.

=============  Exposure Controls/Personal Protection  =============

Respiratory Protection:WEAR NIOSH/MSHA APPROVED RESPIRATOR (OR

EQUIVALENT) SUITABLE FOR CONCENTRATIONS & TYPE OF AIR CONTAMINANTS

ENCOUNTERED.

Ventilation:MECHANICAL LOCAL EXHAUST VENTILATION AT POINT OF

CONTAMINANT RELEASE.

Protective Gloves:IMPERVIOUS GLOVES.

Eye Protection:ANSI APRV CHEM WORK GOGG .

Other Protective Equipment:SAFETY SHOWER, EYEWASH FACILITY WHICH MEET

ANSI DESIGN CRITERIA & PROTECTIVE CLOTHING.

Work Hygienic Practices:WASH EXPOSED AREAS THOROUGHLY W/SOAP & WATER

AFTER HANDLING THIS MATERIAL.

Supplemental Safety and Health

NONE SPECIFIED BY MANUFACTURER.

==================  Physical/Chemical Properties  ==================

HCC:V7

Vapor Pres:<1 @ 20C

Vapor Density:>1

Spec Gravity:0.7 (H*2O=1)

Evaporation Rate & Reference:<1 (BUTYL ACETATE=1)

Solubility in Water:SLIGHT

Appearance and Odor:REDDISH BROWN COLOR PASTE; SLIGHT ODOR.

Percent Volatiles by Volume:<1

=================  Stability and Reactivity Data  =================

Stability Indicator/Materials to Avoid:	YES—Acids, Amines, Strong Bases

Stability Condition to Avoid:		Extreme heat.

Hazardous Decomposition Products:	Carbon monoxide, carbon dioxide, aldehydes

====================  Disposal Considerations  ====================

Waste Disposal Methods: 		INCINERATE IN APPROVED EQUIPMENT. LANDFILL OR

INCINERATE CONTAMINATED MATERIAL I/A/W LOCAL, STATE & FEDERAL REGULATIONS.

Disclaimer (provided with this information by the compiling agencies):

This information is formulated for use by elements of the Department of Defense.  The United States of America in no manner whatsoever, expressly or implied, warrants this information to be accurate and

disclaims all liability for its use.  Any person utilizing this document should seek competent professional advice to verify and assume responsibility for the suitability of this information to their

particular situation.