Henkel Alodine #22 Toner

=====================  Product Identification  =====================

Product ID:ALODINE 22 TONER

MSDS Date: 9/12/2006

Supplier Name: 		DGR Industrial Products, Inc.

4435 North First Street, Suite #184

Livermore, California 94551

Info Phone Num: 	(408) 221-7122

Emergency Phone Num: 	(800) 473-6173

=============  Composition/Information on Ingredients  =============

Ingredient Name: 	Sodium Nitrate

CAS: 			7631-99-4

RTECS #:		WC5600000

Fraction by Wt: 	10-30%

OSHA PEL:		N/K

ACGIH TLV:		N/K

Ingredient Name: 	Sodium Hydroxide

CAS:			1310-73-2

RTECS #:		WB4900000

Fraction by Wt: 	1-10%

OSHA PEL:		2 MG/M3

ACGIH TLV:		2 MG/M3, C

EPA Rpt Qty:		1000 LBS

DOT Rpt Qty:		1000 LBS

=====================  Hazards Identification  =====================

LD50 LC50 Mixture: NONE SPECIFIED BY MANUFACTURER.

Routes of Entry:  Inhalation: YES  Skin: YES  Ingestion: YES

Reports of Carcinogenicity: NTP:NO    IARC:NO	OSHA:NO

Health Hazards Acute and Chronic: EYES: CONTACT WITH EYES WILL CAUSE

SEVERE BURN AND POSSIBLE BLINDNESS. SKIN: CONTACT WITH SKIN OR

MUCOUS MEMBRANE WILL CAUSE SEVERE BURNS AND POSSIBLE ULCERATION.

ABSORPTION NOT EXPECTED. INHALATION:  MIST CAN CAUSE INJURY (BURNS)

TO THE RESPIRATORY TRACT. INGESTION: CAN RESULT IN GASTROINTESTINAL

(EFTS OF OVEREXP)

Explanation of Carcinogenicity: NOT RELEVANT

Effects of Overexposure: HLTH HAZ: DAMAGE; BURNS OF THE DIGESTIVE TRACT.

CHRONIC EFFECTS: NONE EXPECTED.

Medical Conditions Aggravated by Exposure: PRE-EXISTING EYE, SKIN AND

RESPIRATORY DISORDERS.

=======================  First Aid Measures  =======================

First Aid:  SKIN: WASH EXPOS AREA W/PLENTY OF SOAP & WATER. REPEAT

WASHING. RINSE EXPOS AREA FOR AT LEAST 15 MIN. CONSULT HLTH CARE

PROFESSIONAL. IF CLTHG COMES IN CNTCT W/PROD, CLTHG SHOULD BE

REMOVED IMMED & BE	LAUNDERED BEFORE REUSE. EYES: IMMED FLUSH IN

STREAM OF WATER FOR AT LEAST 15 MIN WHILE FORCIBLY HOLDING LIDS

APART TO ENSURE THORO RINSING OF ALL EYE & LID TISS. GET MED ATTN.

INHAL: IMMED REMOVE TO FRESH AIR. IF INDIVIDUAL HAS DFCLTY BRTHG

OR IS CYANOTIC ADMIN OXYGEN IF AVAIL. IF RESPIRATION STOPS, GIVE

MOUTH TO MOUTH RESUSCITATION. GET MED ATTN. INGEST: DO NOT INDUCE

VOMIT. RINSE MOUTH W/COPIOU S AMTS OF WATER FIRST. THEN (SUP DAT)

=====================  Fire Fighting Measures  =====================

Extinguishing Media: AS REQUIRED TO EXTINGUISH SURROUNDING FIRE.

Fire Fighting Procedures: USE NIOSH/MSHA APPROVED SCBA AND FULL

PROTECTIVE EQUIPMENT .

Unusual Fire/Explosion Hazard: NONE.

==================  Accidental Release Measures  ==================

Spill Release Procedures: WEAR PROT CLTHG. INITIALLY MIN AREA AFFECTED

BY SPILL/LEAK. BLOCK ANY POTNTL ROUTES TO WATER SYS. ASSESS IMPACT

ON CONTAMD ENVIRONS & MIN ANY ADVERSE EFTS. CNTCT MFR REP IMMED.

DETERMINE IF FED, STATE  &/OR LOC RELS NOTIFICATION IS REQD.

RECOVER AS MUCH OF PURE PROD AS POSS INTO APPROP CNTNRS. CLAY, SOIL

OR COMMERCIALLY AVAIL ABSORBS MAY BE USED TO RECOVER ANY MATL THAT

CANNOT READILY BE (SUP DAT)

Neutralizing Agent: NONE SPECIFIED BY MANUFACTURER.

======================	Handling and Storage  ======================

Handling and Storage Precautions: KEEP CONTAINERS TIGHTLY CLOSED WHEN

NOT IN USE. FOLLOW NORMAL PROCEDURES FOR SAFE INDUSTRIAL HANDLING

AND STORAGE.

Other Precautions: NEVER ADD WATER TO PRODUCT. WHEN MAKING ADDITIONS TO

HOT SOLUTION, ADD CHEMICAL TO COLD WATER WITH STIRRING, COOL OR

DILUTE AND POUR SLOWLY INTO THE HOT SOLUTION WHILE STIRRING THE

SOLUTION.

=============  Exposure Controls/Personal Protection  =============

Respiratory Protection: NONE REQUIRED UNLESS LARGE AMOUNTS ARE HANDLED

IN AN EMERGENCY OR IF MISTY CONDITIONS EXIST. IN SUCH SITUATIONS, A

NIOSH/MSHA APPROVED GAS MASK SHOULD BE WORN.

Ventilation: USE WITH ADEQUATE VENTILATION TO MINIMIZE EMPLOYEE EXPOSURE TO ANY MIST ABOVE PERMISSIBLE EXPOSURE LIMITS.

Protective Gloves: IMPERVIOUS GLOVES.

Eye Protection: ANSI APPRVD CHEM WORKERS GOGGS AND FULL LENGTH

FACESHIELD .

Other Protective Equipment: ANSI APPROVED EYE WASH & DELUGE SHOWER .

WEAR RUBBER OR CHEMICAL RESISTANT APRON AND SAFETY BOOTS.

Work Hygienic Practices: NONE SPECIFIED BY MANUFACTURER.

Supplemental Safety and Health

FIRST AID PROC: DRINK LG QTYS OF WATER. IF VOMIT OCCURS, DRINK MORE

WATER. GET MED ATTN. IN CASES WHERE INDIVIDUAL IS SEMICONSCIOUS,

COMATOSE OR CONVULSING, DO NOT GIVE FLUIDS BY MOUTH. CONTACT MD IMM

EDIATELY. SPILL PROC: RECOVERED AS PURE PRODUCT. CONTACT PERSON(S)

RESPONSIBLE FOR OPERATION OF FACILITY'S (OTHER INFO)

==================  Physical/Chemical Properties  ==================

Boiling Pt: 		B.P. Text:>210F,>99C

Spec Gravity:		1.1-1.2

pH:			>12.5

Solubility in Water: 	COMPLETE

Appearance and Odor: 	CLEAR, COLORLESS LIQUID; NO ODOR.

=================  Stability and Reactivity Data  =================

Stability Indicator/Materials to Avoid:	YES

KEEP SEPARATE FROM ACIDS. WATER ADDED TO THIS CHEMICAL

MAY CAUSE LOCALIZED OVERHEATING AND SPLATTERING.

Stability Condition to Avoid:			NONE EXPECTED.

Hazardous Decomposition Products:		NITROGEN OXIDES.

====================  Disposal Considerations  ====================

Waste Disposal Methods: FOLLOW ALL FED, STATE & LOC REGS GOVERNING DISP

OF WASTE MATLS. DETERM IF WASTE CNTNG PROD CAN BE HANDLED BY AVAIL

INDUS EFFLUENT SYS OR OTHER ON-SITE WASTE MANAGEMENT. PROD IS NOT

SPECIFICALLY LISTED IN 40 CFR 261 AS RCRA HAZ WASTE. HOWEVER SPILL

OR LEAK RESIDUALS MAY MEET CRITERIA OF CHARACT HAZ W ASTE UNDER

THIS ACT. CNTNR DISP: MFR (OTHER INFO)

=======================  Other Information  =======================

Disclaimer (provided with this information by the compiling agencies):

This information is formulated for use by elements of the Department

of Defense.  The United States of America in no manner whatsoever,

expressly or implied, warrants this information to be accurate and

disclaims all liability for its use.  Any person utilizing this

document should seek competent professional advice to verify and

assume responsibility for the suitability of this information to their

particular situation.