MSDS Number: G6820 * * * * * Effective Date: 11/09/06 * * * * * Supercedes: 02/12/04

# GLYCOLIC ACID (68 - 72%)

## 1. Product Identification

Synonyms: Hydroxyacetic acid; hydroxyethanoic acid 
CAS No.: 79-14-1 
Molecular Weight: Not applicable to mixtures. 
Chemical Formula: HOCH2COOH in H2O 
Product Codes: 
J.T. Baker: M822 
Mallinckrodt: 2742

## 2. Composition/Information on Ingredients

Ingredient                    CAS No         Percent        Hazardous

------------------------   ------------   ------------      ---------

Glycolic Acid              79-14-1          68 - 72%        Yes

Water                      7732-18-5        28 - 32%        No

## 3. Hazards Identification

Emergency Overview 
-------------------------- 
DANGER! CORROSIVE. HARMFUL IF SWALLOWED OR INHALED. CAUSES SEVERE IRRITATION AND BURNS TO EVERY AREA OF CONTACT. 

SAF-T-DATA(tm) Ratings (Provided here for your convenience) 
----------------------------------------------------------------------------------------------------------- 
Health Rating: 3 - Severe 
Flammability Rating: 1 - Slight 
Reactivity Rating: 1 - Slight 
Contact Rating: 4 - Extreme (Corrosive) 
Lab Protective Equip: GOGGLES & SHIELD; LAB COAT & APRON; VENT HOOD; PROPER GLOVES 
Storage Color Code: White (Corrosive) 
----------------------------------------------------------------------------------------------------------- 

Potential Health Effects 
---------------------------------- 
Information on the human health effects from exposure to this substance is limited. 
Inhalation: 
Inhalation is irritating to the mucous membrane and upper respiratory tract. In extreme cases, severe irritation and burns may result. 
Ingestion: 
Material causes irritation and burns to digestive tract. 
Skin Contact: 
Causes irritation and burns to the skin. 
Eye Contact: 
Can cause severe irritation or burns with eye damage. 
Chronic Exposure: 
No information found. 
Aggravation of Pre-existing Conditions: 
No information found.

## 4. First Aid Measures

Inhalation: 
Remove to fresh air. If not breathing, give artificial respiration. If breathing is difficult, give oxygen. Call a physician. 
Ingestion: 
DO NOT INDUCE VOMITING. Give large quantities of water. Never give anything by mouth to an unconscious person. Call a physician immediately. 
Skin Contact: 
In case of contact, immediately flush skin with plenty of water for at least 15 minutes while removing contaminated clothing and shoes. Wash clothing before reuse. Call a physician. 
Eye Contact: 
Immediately flush eyes with plenty of water for at least 15 minutes, lifting lower and upper eyelids occasionally. Get medical attention immediately.

## 5. Fire Fighting Measures

Fire: 
Not considered to be a fire hazard. 
Explosion: 
Not considered to be an explosion hazard. 
Fire Extinguishing Media: 
Water spray, dry chemical, alcohol foam, or carbon dioxide. 
Special Information: 
In the event of a fire, wear full protective clothing and NIOSH-approved self-contained breathing apparatus with full facepiece operated in the pressure demand or other positive pressure mode.

## 6. Accidental Release Measures

Ventilate area of leak or spill. Wear appropriate personal protective equipment as specified in Section 8. Isolate hazard area. Keep unnecessary and unprotected personnel from entering. Contain and recover liquid when possible. Neutralize with alkaline material (soda ash, lime), then absorb with an inert material (e. g., vermiculite, dry sand, earth), and place in a chemical waste container. Do not use combustible materials, such as saw dust. Do not flush to sewer!

## 7. Handling and Storage

Keep in a tightly closed container, stored in a cool, dry, ventilated area. Protect against physical damage. Isolate from incompatible substances. Containers of this material may be hazardous when empty since they retain product residues (vapors, liquid); observe all warnings and precautions listed for the product.

## 8. Exposure Controls/Personal Protection

Airborne Exposure Limits: 
None established. 
Ventilation System: 
A system of local and/or general exhaust is recommended to keep employee exposures as low as possible. Local exhaust ventilation is generally preferred because it can control the emissions of the contaminant at its source, preventing dispersion of it into the general work area. Please refer to the ACGIH document, Industrial Ventilation, A Manual of Recommended Practices, most recent edition, for details. 
Personal Respirators (NIOSH Approved): 
For conditions of use where exposure to the substance is apparent and engineering controls are not feasible, consult an industrial hygienist. For emergencies, or instances where the exposure levels are not known, use a full-facepiece positive-pressure, air-supplied respirator. WARNING: Air purifying respirators do not protect workers in oxygen-deficient atmospheres. 
Skin Protection: 
Wear impervious protective clothing, including boots, gloves, lab coat, apron or coveralls, as appropriate, to prevent skin contact. 
Eye Protection: 
Use chemical safety goggles and/or full face shield where dusting or splashing of solutions is possible. Maintain eye wash fountain and quick-drench facilities in work area.

## 9. Physical and Chemical Properties

Appearance: 					White to straw colored liquid. 
Odor: 						Burnt sugar odor 
Solubility: 					Soluble in water. 
Specific Gravity: 				1.27 
pH: 						Aqueous solution: 2.5 (0.5 %), 1.7 (10 %) 
% Volatiles by volume @ 21C (70F): 	No information found. 
Boiling Point: 				No information found. 
Melting Point: 				10C (50F) 
Vapor Density (Air=1): 			No information found. 
Vapor Pressure (mm Hg): 			No information found. 
Evaporation Rate (BuAc=1): 		No information found.

## 10. Stability and Reactivity

Stability: 
Stable under ordinary conditions of use and storage. 
Hazardous Decomposition Products: 
Carbon dioxide and carbon monoxide may form when heated to decomposition. Can react with most common metals to release flammable hydrogen gas. 
Hazardous Polymerization: 
Will not occur. 
Incompatibilities: 
Strong oxidizers. 
Conditions to Avoid: 
Incompatibles.

## 11. Toxicological Information

For Glycolic Acid: Oral rat LD50: 1950 mg/Kg; Irritation Data (std Draize, eye, rabbit) 2 mg, severe. Investigated as a tumorigen and a reproductive effector.

-\Cancer Lists\------------------------------------------------------

---NTP Carcinogen---

Ingredient                     Known    Anticipated    IARC Category

----------------------------   -----    -----------    -------------

Glycolic Acid (79-14-1)          No          No            None

Water (7732-18-5)                No          No            None

## 12. Ecological Information

Environmental Fate: 		No information found. 
Environmental Toxicity: 		No information found.

## 13. Disposal Considerations

Whatever cannot be saved for recovery or recycling should be handled as hazardous waste and sent to a RCRA approved incinerator or disposed in a RCRA approved waste facility. Processing, use or contamination of this product may change the waste management options. State and local disposal regulations may differ from federal disposal regulations. Dispose of container and unused contents in accordance with federal, state and local requirements.

## 14. Transport Information

Domestic (Land, D.O.T.) 
----------------------- 
Proper Shipping Name: CORROSIVE LIQUID, ACIDIC, ORGANIC, N.O.S. (GLYCOLIC ACID) 
Hazard Class: 8 
UN/NA: UN3265 
Packing Group: II 
Information reported for product/size: 500ML 

International (Water, I.M.O.) 
----------------------------- 
Proper Shipping Name: CORROSIVE LIQUID, ACIDIC, ORGANIC, N.O.S. (GLYCOLIC ACID) 
Hazard Class: 8 
UN/NA: UN3265 
Packing Group: II 
Information reported for product/size: 500ML 

International (Air, I.C.A.O.) 
----------------------------- 
Proper Shipping Name: CORROSIVE LIQUID, ACIDIC, ORGANIC, N.O.S. (GLYCOLIC ACID) 
Hazard Class: 8 
UN/NA: UN3265 
Packing Group: II 
Information reported for product/size: 500ML

## 15. Regulatory Information

-\Chemical Inventory Status - Part 1\---------------------------------

Ingredient                              TSCA  EC   Japan  Australia

--------------------------------------  ----  ---  -----  ---------

Glycolic Acid (79-14-1)                Yes  Yes   Yes      Yes

Water (7732-18-5)                      Yes  Yes   Yes      Yes

-\Chemical Inventory Status - Part 2\---------------------------------

--Canada--

Ingredient                              Korea  DSL   NDSL  Phil.

--------------------------------------  -----  ---   ----  -----

Glycolic Acid (79-14-1)                Yes   Yes   No     Yes

Water (7732-18-5)                      Yes   Yes   No     Yes

-\Federal, State & International Regulations - Part 1\----------------

-SARA 302-    ------SARA 313------

Ingredient                         RQ    TPQ     List  Chemical Catg.

----------------------------------  ---   -----   ----  --------------

Glycolic Acid (79-14-1)            No    No      No         No

Water (7732-18-5)                  No    No      No         No

-\Federal, State & International Regulations - Part 2\----------------

-RCRA-    -TSCA-

Ingredient                              CERCLA     261.33     8(d)

--------------------------------------  ------     ------    ------

Glycolic Acid (79-14-1)                   No         No         No

Water (7732-18-5)                         No         No         No

Chemical Weapons Convention:  No     TSCA 12(b):  No     CDTA:  No

SARA 311/312:  Acute: Yes      Chronic: No   Fire: No  Pressure: No

Reactivity: No          (Mixture / Liquid)

Australian Hazchem Code: None allocated. 
Poison Schedule: None allocated. 
WHMIS: 
This MSDS has been prepared according to the hazard criteria of the Controlled Products Regulations (CPR) and the MSDS contains all of the information required by the CPR.

## 16. Other Information

NFPA Ratings: Health: 3 Flammability: 1 Reactivity: 0 
Label Hazard Warning: 
DANGER! CORROSIVE. HARMFUL IF SWALLOWED OR INHALED. CAUSES SEVERE IRRITATION AND BURNS TO EVERY AREA OF CONTACT. 
Label Precautions: 
Do not breathe vapor or mist.
Do not get in eyes, on skin, or on clothing.
Keep container closed.
Use only with adequate ventilation.
Wash thoroughly after handling. 
Label First Aid: 
In case of contact, immediately flush eyes or skin with plenty of water for at least 15 minutes. Remove contaminated clothing and shoes. Wash clothing before reuse. Call a physician. If inhaled, remove to fresh air. If not breathing, give artificial respiration. If breathing is difficult, give oxygen. Call a physician. If swallowed, DO NOT INDUCE VOMITING. Give large quantities of water. Never give anything by mouth to an unconscious person. Call a physician immediately. 
Product Use: 
Laboratory Reagent. 
Revision Information: 
MSDS Section(s) changed since last revision of document include: 3. 
Disclaimer: 
************************************************************************ 
Mallinckrodt Baker, Inc. provides the information contained herein in good faith but makes no representation as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose. MALLINCKRODT BAKER, INC. MAKES NO REPRESENTATIONS OR WARRANTIES, EITHER EXPRESS OR IMPLIED, INCLUDING WITHOUT LIMITATION ANY WARRANTIES OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE WITH RESPECT TO THE INFORMATION SET FORTH HEREIN OR THE PRODUCT TO WHICH THE INFORMATION REFERS. ACCORDINGLY, MALLINCKRODT BAKER, INC. WILL NOT BE RESPONSIBLE FOR DAMAGES RESULTING FROM USE OF OR RELIANCE UPON THIS INFORMATION. 
************************************************************************ 
Prepared by: Environmental Health & Safety
Phone Number: (314) 654-1600 (U.S.A.)