MSDS Number: A6108 * * * * * Effective Date: 08/02/07 * * * * * Supercedes: 12/14/04

# AMMONIUM PHOSPHATE MONOBASIC

## 1. Product Identification

Synonyms: Primary ammonium phosphate; ammonium dihydrogen phosphate; ammonium biphosphate; phosphoric acid, monoammonium salt 
CAS No.: 7722-76-1 
Molecular Weight: 115.04 
Chemical Formula: NH4H2PO4 
Product Codes: 
J.T. Baker: 0776, 0777, 4931 
Mallinckrodt: 3476

## 2. Composition/Information on Ingredients

Ingredient                     CAS No         Percent      Hazardous

--------------------------   ------------   ------------   ---------

Ammonium Phosphate Monobasic  7722-76-1       98 - 100%        Yes

## 3. Hazards Identification

Emergency Overview 
-------------------------- 
WARNING! CAUSES IRRITATION TO SKIN, EYES AND RESPIRATORY TRACT. HARMFUL IF SWALLOWED OR INHALED. 

SAF-T-DATA(tm) Ratings (Provided here for your convenience) 
----------------------------------------------------------------------------------------------------------- 
Health Rating: 1 - Slight 
Flammability Rating: 0 - None 
Reactivity Rating: 1 - Slight 
Contact Rating: 2 - Moderate 
Lab Protective Equip: GOGGLES; LAB COAT; PROPER GLOVES 
Storage Color Code: Green (General Storage) 
----------------------------------------------------------------------------------------------------------- 

Potential Health Effects 
---------------------------------- 

Inhalation: 
Causes irritation to the respiratory tract. Symptoms may include coughing, shortness of breath. 
Ingestion: 
Causes irritation to the gastrointestinal tract. Symptoms may include nausea, vomiting and diarrhea. 
Skin Contact: 
Causes irritation to skin. Symptoms include redness, itching, and pain. 
Eye Contact: 
Causes irritation, redness, and pain. 
Chronic Exposure: 
No information found. 
Aggravation of Pre-existing Conditions: 
No information found.

## 4. First Aid Measures

Inhalation: 
Remove to fresh air. If not breathing, give artificial respiration. If breathing is difficult, give oxygen. Get medical attention. 
Ingestion: 
Induce vomiting immediately as directed by medical personnel. Never give anything by mouth to an unconscious person. Get medical attention. 
Skin Contact: 
Immediately flush skin with plenty of water for at least 15 minutes. Remove contaminated clothing and shoes. Get medical attention. Wash clothing before reuse. Thoroughly clean shoes before reuse. 
Eye Contact: 
Immediately flush eyes with plenty of water for at least 15 minutes, lifting upper and lower eyelids occasionally. Get medical attention.

## 5. Fire Fighting Measures

Fire: 
Not considered to be a fire hazard. Fire may produce toxic fumes. 
Explosion: 
Not considered to be an explosion hazard. 
Fire Extinguishing Media: 
Use any means suitable for extinguishing surrounding fire. 
Special Information: 
In the event of a fire, wear full protective clothing and NIOSH-approved self-contained breathing apparatus with full facepiece operated in the pressure demand or other positive pressure mode.

## 6. Accidental Release Measures

Ventilate area of leak or spill. Wear appropriate personal protective equipment as specified in Section 8. Spills: Pick up and place in a suitable container for reclamation or disposal, using a method that does not generate dust.

## 7. Handling and Storage

Keep in a tightly closed container, stored in a cool, dry, ventilated area. Protect against physical damage. Isolate from incompatible substances. Containers of this material may be hazardous when empty since they retain product residues (dust, solids); observe all warnings and precautions listed for the product.

## 8. Exposure Controls/Personal Protection

Airborne Exposure Limits: 
None established. 
Ventilation System: 
In general, dilution ventilation is a satisfactory health hazard control for this substance. However, if conditions of use create discomfort to the worker, a local exhaust system should be considered. 
Personal Respirators (NIOSH Approved): 
For conditions of use where exposure to dust or mist is apparent and engineering controls are not feasible, a particulate respirator (NIOSH type N95 or better filters) may be worn. If oil particles (e.g. lubricants, cutting fluids, glycerine, etc.) are present, use a NIOSH type R or P filter. For emergencies or instances where the exposure levels are not known, use a full-face positive-pressure, air-supplied respirator. WARNING: Air-purifying respirators do not protect workers in oxygen-deficient atmospheres. 
Skin Protection: 
Wear impervious protective clothing, including boots, gloves, lab coat, apron or coveralls, as appropriate, to prevent skin contact. 
Eye Protection: 
Use chemical safety goggles and/or full face shield where dusting or splashing of solutions is possible. Maintain eye wash fountain and quick-drench facilities in work area.

## 9. Physical and Chemical Properties

Appearance: 
Brilliant white crystals or granules. 
Odor: 
Faint acid odor. 
Solubility: 
1 g/2.5 ml water. 
Specific Gravity: 
1.80 
pH: 
4.2 
% Volatiles by volume @ 21C (70F): 
0 
Boiling Point: 
No information found. 
Melting Point: 
190C (374F) 
Vapor Density (Air=1): 
No information found. 
Vapor Pressure (mm Hg): 
No information found. 
Evaporation Rate (BuAc=1): 
No information found.

## 10. Stability and Reactivity

Stability: 
Stable under ordinary conditions of use and storage. 
Hazardous Decomposition Products: 
Burning may produce ammonia and oxides of phosphorus. 
Hazardous Polymerization: 
Will not occur. 
Incompatibilities: 
Sodium hypochlorite. 
Conditions to Avoid: 
Incompatibles.

## 11. Toxicological Information

No LD50/LC50 information found relating to normal routes of occupational exposure.

-\Cancer Lists\------------------------------------------------------

---NTP Carcinogen---

Ingredient                    Known    Anticipated    IARC Category

---------------------------   -----    -----------    -------------

Ammonium Phosphate Monobasic    No          No            None

(7722-76-1)

## 12. Ecological Information

Environmental Fate: 
No information found. 
Environmental Toxicity: 
No information found.

## 13. Disposal Considerations

Whatever cannot be saved for recovery or recycling should be managed in an appropriate and approved waste disposal facility. Processing, use or contamination of this product may change the waste management options. State and local disposal regulations may differ from federal disposal regulations. Dispose of container and unused contents in accordance with federal, state and local requirements.

## 14. Transport Information

Not regulated.

## 15. Regulatory Information

-\Chemical Inventory Status - Part 1\---------------------------------

Ingredient                                TSCA  EC   Japan  Australia

----------------------------------------  ----  ---  -----  ---------

Ammonium Phosphate Monobasic (7722-76-1)  Yes  Yes   Yes      Yes

-\Chemical Inventory Status - Part 2\---------------------------------

--Canada--

Ingredient                                  Korea  DSL   NDSL  Phil.

------------------------------------------  -----  ---   ----  -----

Ammonium Phosphate Monobasic (7722-76-1)     Yes   Yes   No     Yes

-\Federal, State & International Regulations - Part 1\----------------

-SARA 302-    ------SARA 313------

Ingredient                           RQ    TPQ     List  Chemical Catg.

---------------------------------- ---   -----    ----  --------------

Ammonium Phosphate Monobasic(7722-76-1)No  No      No         No

-\Federal, State & International Regulations - Part 2\----------------

-RCRA-    -TSCA-

Ingredient                                CERCLA     261.33     8(d)

----------------------------------------  ------     ------    ------

Ammonium Phosphate Monobasic               No         No         No

(7722-76-1)

Chemical Weapons Convention:  No     TSCA 12(b):  No     CDTA:  No

SARA 311/312:  Acute: Yes      Chronic: No   Fire: No  Pressure: No

Reactivity: No          (Pure / Solid)

Australian Hazchem Code: None allocated. 
Poison Schedule: None allocated. 
WHMIS: 
This MSDS has been prepared according to the hazard criteria of the Controlled Products Regulations (CPR) and the MSDS contains all of the information required by the CPR.

## 16. Other Information

NFPA Ratings: Health: 2 Flammability: 0 Reactivity: 0 
Label Hazard Warning: 
WARNING! CAUSES IRRITATION TO SKIN, EYES AND RESPIRATORY TRACT. HARMFUL IF SWALLOWED OR INHALED. 
Label Precautions: 
Avoid contact with eyes, skin and clothing.
Avoid breathing dust.
Wash thoroughly after handling.
Keep container closed.
Use only with adequate ventilation. 
Label First Aid: 
In case of contact, immediately flush eyes or skin with plenty of water for at least 15 minutes. Remove contaminated clothing and shoes. Wash clothing before reuse. If inhaled, remove to fresh air. If not breathing, give artificial respiration. If breathing is difficult, give oxygen. If swallowed, induce vomiting immediately as directed by medical personnel. Never give anything by mouth to an unconscious person. In all cases, get medical attention. 
Product Use: 
Laboratory Reagent. 
Revision Information: 
No Changes. 
Disclaimer: 
************************************************************************ 
Mallinckrodt Baker, Inc. provides the information contained herein in good faith but makes no representation as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose. MALLINCKRODT BAKER, INC. MAKES NO REPRESENTATIONS OR WARRANTIES, EITHER EXPRESS OR IMPLIED, INCLUDING WITHOUT LIMITATION ANY WARRANTIES OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE WITH RESPECT TO THE INFORMATION SET FORTH HEREIN OR THE PRODUCT TO WHICH THE INFORMATION REFERS. ACCORDINGLY, MALLINCKRODT BAKER, INC. WILL NOT BE RESPONSIBLE FOR DAMAGES RESULTING FROM USE OF OR RELIANCE UPON THIS INFORMATION. 
************************************************************************ 
Prepared by: Environmental Health & Safety
Phone Number: (314) 654-1600 (U.S.A.)