MATERIAL SAFETY DATA SHEET

I            PRODUCT IDENTIFICATION

Trade Name:      Potassium                                                                                    Formula:            K

CAS #:                7440-09-7

II           HAZARDOUS INGREDIENTS

Hazardous Component                  %                        OSHA/PEL                       ACGIH/TLV

Potassium                                         0-100                   N/E                                    N/E

Component subject to reporting of the section 313 of the Emergency Planning and Community Right-to-Know Act: No Data

Title Compound: Not listed.

III         PHYSICAL DATA

Boiling Point:                   760˚C                                                            Melting Point:                  63.7˚C

Specific Gravity:              0.86                                                                Solubility in H2O:            Reacts violently with water

Vapor Pressure:               .0899 mm (245˚C)                                        Color and Form:              Under argon

IV         FIRE AND EXPLOSION HAZARDS DATA

Flash Point (Method used): No data                                                      Autoignition Temperature: N/A

Extinguishing Media: Dry powder only

Special Firefighting Procedures: Highly flammable. Material readily ignited with flame, sparks, friction or heat. Burning material may release toxic and corrosive fumes. Leave the area unless fitted with a self-contained breathing apparatus and fire protective clothing.

Unusual Fire and Explosion Hazard: Pyrophoric material. Material may ignite spontaneously on contact with air.

V           HEALTH HAZARD INFORMATION

Effects of Exposure: Contact with skin can cause severe chemical burns.

Route(s) of Entry: Skin, ingestion

Carcinogenicity: NTP: No           IARC: No          OSHA: No

EMERGENCY AND FIRST AID PROCEDURES:

INHALATION: If inhaled remove victim to fresh air and seek medical attention if coughing, shortness of breath or irritation persists.

INGESTION: If ingested give victim plenty of water and seek immediate medical assistance.

SKIN: In case of skin contact wash affected area with soap and water for at least 5 minutes.

EYES: In case of eye contact immediately flush the eyes with water for at least 15 minutes.

VI         REACTIVITY DATA

Stability: Air sensitive, moisture sensitive.

Conditions to Avoid: Contact with air or moisture. Reacts explosively with water.

Incompatibility (Material to Avoid): Water, air, oxidizers, halogens, sulfur, phosphorus, carbon, alcohols, halogenated hydrocarbons, carbonyl compounds and carbon dioxide.

Hazardous Decomposition Products: Potassium oxide and potassium superoxide.

Hazardous Polymerization: Will not occur

VII        SPILL OR LEAK PROCEDURES

Steps to be Taken in Case Material is Released or Spilled: The material will ignite spontaneously in air. Burning material may release toxic fumes. Leave the area unless fitted with a self-contained breathing apparatus. Smother fire with dry powder. Do not use water.

Waste Disposal Method: Material should be disposed of in accordance with local, state and federal regulations.

VIII      SPECIAL PROTECTION INFORMATION

Respiratory Protection (Specify Type): Wear a NIOSH/MSHA approved dust mask.

Ventilation: Use local exhaust

Protective Gloves: Yes

Eye Protection: Yes

Other Protective Equipment: Wear a full face shield, lab apron and suitable gloves.

IX         SPECIAL PRECAUTIONS

Precautions to be Taken in Handling and Storage: Handle and store the material under an inert atmosphere of nitrogen or argon. Keep away from heat.

Other Precautions: The material will react with air and moisture. Prolonged contact with air may result in the formation of potassium superoxide, a potentially explosive compound.

Hazard Class: 4.3            Sub-Risk: N/A                 Packing Group: I            UN ID Number: UN# 2257

The above information is believed to be correct, but does not purport to be all inclusive and shall be used only as a guide. ESPI shall not be held liable for any damages resulting from handling or from contact with the above product.