Material Safety Data Sheet

Butylated Hydroxytoluene

Section 1. Product and Company Identification

Product name    	Butylated Hydroxytoluene

Synonyms           	Butylhydroxytoluene,

2,6-di-t-butyl-4-methylphenol

Physical state     	Solid (Crystalline powder)

Odor                   	Slight

Supplier

DGR Industrial Products, Inc.

4435 North First Street, Suite #184

Livermore, California 94551

(408) 221-7122

www.chemical-supermarket.com

OSHA/HCS status	This material is considered hazardous by the OSHA Hazard Communication Standard (29 CFR 1910.1200).

Emergency overview

WARNING!

HARMFUL IF INHALED OR SWALLOWED.
CAUSES RESPIRATORY TRACT, EYE AND SKIN IRRITATION.
CAUSES DAMAGE TO THE FOLLOWING ORGANS: SKIN, EYES, EYE, LENS OR CORNEA.

Do not ingest. Avoid contact with skin and clothing. Avoid breathing dust. Keep container closed. Use only with adequate ventilation. Wash thoroughly after handling.

Routes of entry

Dermal contact. Eye contact. Inhalation. Ingestion.

Section 2. Hazards Identification

Potential acute health effects

Eyes		Irritating to eyes.

Skin		Irritating to skin

Inhalation		Toxic by inhalation. Irritating to respiratory system.

Ingestion		Toxic if swallowed.

Carcinogenic effects	No known significant effects or critical hazards.

Mutagenic effects	No known significant effects or critical hazards.

Teratogenicity / Reproductive toxicity	No known significant effects or critical hazards.

Medical conditions aggravated by over-exposure

Repeated skin exposure can produce local skin destruction or dermatitis. Repeated or prolonged exposure to the substance can produce lung damage. Repeated exposure of the eyes to a low level of dust can produce eye irritation. Repeated or prolonged exposure to the substance can produce target organs damage.

In case of emergency

800-424-9300 CHEMTREC (USA)
613-996-6666 CANUTEC (Canada)
24 Hours/Day: 7 Days/Week

Section 3. Composition/Information on Ingredients

Name						CAS number		% by Weight

Butylated Hydroxytoluene				128-37-0			100

Section 4. First Aid Measures

Eye contact	Immediately flush eyes with plenty of water for at least 15 minutes, occasionally lifting the upper and lower eyelids. Check for and remove any contact lenses. Get medical attention.

Skin contact		Get medical attention immediately. Flush contaminated skin with plenty of water. Continue to rinse

for at least 10 minutes. Remove contaminated clothing and shoes. Wash contaminated clothing

thoroughly with water before removing or wear gloves. Wash clothing before reuse. Clean shoes

thoroughly before reuse.

Inhalation			Get medical attention immediately. Move exposed person to fresh air. If it is suspected that fumes

are still present, the rescuer should wear an appropriate mask or self-contained breathing apparatus.

Keep person warm and at rest. If not breathing, if breathing is irregular or if respiratory arrest occurs,

provide artificial respiration or oxygen by trained personnel. It may be dangerous to the person

providing aid to give mouth-to-mouth resuscitation. If unconscious, place in recovery position and

get medical attention immediately. Maintain an open airway. Loosen tight clothing such as a collar,

tie, belt or waistband.

Ingestion			Get medical attention immediately. Wash out mouth with water. Remove dentures if any. Move

exposed person to fresh air. Keep person warm and at rest. If material has been swallowed and the

exposed person is conscious, give small quantities of water to drink. Stop if the exposed person feels

sick as vomiting may be dangerous. Do not induce vomiting unless directed to do so by medical

personnel. If vomiting occurs, the head should be kept low so that vomit does not enter the lungs.

Never give anything by mouth to an unconscious person. If unconscious, place in recovery position

and get medical attention immediately. Maintain an open airway. Loosen tight clothing such as a

collar, tie, belt or waistband.

Protection of first-aiders	No action shall be taken involving any personal risk or without suitable training. If it is suspected

that fumes are still present, the rescuer should wear an appropriate mask or self-contained breathing

apparatus. It may be dangerous to the person providing aid to give mouth-to-mouth resuscitation.

Section 5. Fire Fighting Measures

Flammability of the product		Combustible when exposed to heat or flame.

Products of combustion		These products are carbon oxides (CO, CO2).

Extinguishing media			Use an extinguishing agent suitable for the surrounding fire.

Special protective equipment for fire-fighters	Fire-fighters should wear appropriate protective equipment and self-contained breathing apparatus (SCBA) with a full face-piece operated in positive pressure mode.

Special remarks on fire hazards	Development of hazardous combustion gases or vapors possible in the event of fire.

Avoid contact with oxidizing substances.

Special remarks on explosion hazards	Avoid all possible sources of ignition (spark or flame).

Section 6. Accidental Release Measures

Personal precautions			Immediately contact emergency personnel. Keep unnecessary personnel away. Use

suitable protective equipment.

Environmental precautions		Avoid dispersal of spilled material and runoff and contact with soil, waterways, drains

and sewers.

Methods for cleaning up		If emergency personnel are unavailable, vacuum or carefully scoop up spilled material

and place in an appropriate container for disposal by incineration. Avoid creating dusty

conditions and prevent wind dispersal.

Section 7. Handling and Storage

Handling		Do not ingest. Avoid contact with eyes, skin and clothing. Keep container closed. Use only with adequate

ventilation. Avoid breathing dust. Wash thoroughly after handling.

Storage		Keep container tightly closed. Keep container in a cool, well-ventilated area.

Section 8. Exposure Controls/Personal Protection

ACGIH TLV (United States, 2000).
TWA: 10 mg/m3 8 hour/hours.
NIOSH REL (United States, 12/2001).
TWA: 10 mg/m3 10 hour/hours.
OSHA Final Rule (United States, 1989).
TWA: 10 mg/m3 8 hour/hours.
OSHA PEL 1989 (United States, 3/1989).
TWA: 10 mg/m3 8 hour/hours.
ACGIH TLV (United States, 1/2006). Notes: 1996 Adoption Refers to Appendix A -- Carcinogens. Inhalable fraction. See Appendix C, paragraph A. Inhalable Particulate Mass TLVs (IPM–TLVs) for those materials that are hazardous when deposited anywhere in the respiratory tract. Vapor and aerosol
TWA: 2 mg/m3 8 hour/hours. Form: Vapor and aerosols

Personal protection

Eyes	Safety eyewear complying with an approved standard should be used when a risk assessment indicates this is necessary to

avoid exposure to liquid splashes, mists, gases or dusts.  Recommended: face shield

Skin	Personal protective equipment for the body should be selected based on the task being performed and the risks involved

and should be approved by a specialist before handling this product.  Body: Recommended: safety apron and gloves

Respiratory	Use a properly fitted, particulate filter respirator complying with an approved standard if a risk assessment

indicates this is necessary. Respirator selection must be based on known or anticipated exposure levels, the

hazards of the product and the safe working limits of the selected respirator.

Hands	Chemical-resistant, impervious gloves complying with an approved standard should be worn at all times when handling

chemical products if a risk assessment indicates this is necessary. Recommended: nitrile rubber.

Hygiene measures	Wash hands, forearms and face thoroughly after handling chemical products, before eating, smoking and using

the lavatory and at the end of the working period. Appropriate techniques should be used to remove potentially

contaminated clothing. Wash contaminated clothing before reusing. Ensure that eyewash stations and safety

showers are close to the workstation location.

Section 9. Physical and Chemical Properties

Physical state		Solid. (Crystalline granules.)

Flash point		Closed cup: 126.85°C (260.3°F).

Auto-ignition temperature	470°C (878°F)

Color			White.

Odor			Slight

Molecular weight		220.39 g/mole

Molecular formula		C15-H24-O

Boiling/condensation point	265°C (509°F)

Melting/freezing point	69°C (156.2°F)

Relative density		1.048 (Water = 1)

Vapor pressure		0.001 kPa (0.01 mm Hg) (at 20°C)

Vapor density		7.6 (Air = 1)

Section 10. Stability and Reactivity

Stability and reactivity		The product is stable.

Conditions of instability		Avoid exposure to heat and light.

Incompatibility with various substances	Reactive or incompatible with the following materials: oxidizing materials, metals, acids

and alkalis.  Avoid all possible sources of ignition (spark or flame). Avoid generating

dust. Incompatible with peroxides, bases, sulfuric acids, and strong acids.

Hazardous decomposition products	carbon oxides (CO, CO2)

Hazardous polymerization		Will not occur.

Conditions of reactivity		Avoid all possible sources of ignition (spark or flame).

Section 11. Toxicological Information

Product/ingredient name	Test	Test		Route		Species

LD50	890 mg/kg		Oral		Rat
LD50	2100 mg/kg	Oral		Rabbit
LD50	650 mg/kg		Oral		Mouse
LDLo	2000 mg/kg	Oral		Rabbit
LDLo	2100 mg/kg	Oral		Rabbit
LDLo	940 mg/kg		Oral		Cat

Chronic effects on humans	CARCINOGENIC EFFECTS: A4 (Not classifiable for humans or animals.) by ACGIH, 3 (Not

classifiable for humans.) by IARC.  Causes damage to the following organs: skin, eyes, eye, lens or

cornea.

Other toxic effects on humans

Very hazardous in case of skin contact (irritant), of eye contact (irritant), of ingestion, of inhalation (lung irritant).

Inhalation			Irritating to respiratory system.

Eyes			Irritating to eyes.

Skin			Irritating to skin.

Section 12. Ecological Information

Environmental precautions		No known significant effects or critical hazards.

Products of degradation		These products are carbon oxides (CO, CO2) and water.

Toxicity of the products of biodegradation	The products of degradation are less toxic than the product itself.

Section 13. Disposal Considerations

Waste disposal	The generation of waste should be avoided or minimized wherever possible. Avoid dispersal of spilled material

and runoff and contact with soil, waterways, drains and sewers. Disposal of this product, solutions and any by-

products should at all times comply with the requirements of environmental protection and waste disposal

legislation and any regional local authority requirements.

Disposal should be in accordance with applicable regional, national and local laws and regulations. Local regulations may be more stringent than regional or national requirements.

The information presented below only applies to the material as supplied. The identification based on characteristic(s) or listing may not apply if the material has been used or otherwise contaminated. It is the responsibility of the waste generator to determine the toxicity and physical properties of the material generated to determine the proper waste identification and disposal methods in compliance with applicable regulations.

Refer to Section 7: HANDLING AND STORAGE and Section 8: EXPOSURE CONTROLS/PERSONAL PROTECTION for additional handling information and protection of employees.

Section 14. Transport Information

Not Regulated

Section 15. Regulatory Information

HCS Classification		Toxic material, Irritating material, Target organ effects

U.S. Federal regulations	TSCA 8(b) inventory: Listed

SARA 302/304/311/312 extremely hazardous substances: No products were found.
SARA 302/304 emergency planning and notification: No products were found.
SARA 302/304/311/312 hazardous chemicals: Butylhydroxytoluene
SARA 311/312 MSDS distribution - chemical inventory - hazard identification: Butylhydroxytoluene: Immediate (acute) health hazard, Delayed (chronic) health hazard

WHMIS (Canada)		Class D-2B: Material causing other toxic effects (Toxic).

Section 16. Other Information

Label requirements		WARNING!

HARMFUL IF INHALED OR SWALLOWED.

CAUSES RESPIRATORY TRACT, EYE AND SKIN IRRITATION.

CAUSES DAMAGE TO THE FOLLOWING ORGANS: SKIN, EYES, EYE, LENS OR CORNEA.

National Fire Protection Association (U.S.A.)

Health:	0

Fire:	1

Reactivity:	0

The statements contained herein are based upon technical data that is believed to be reliable, is offered for information purposes only and as a guide to the appropriate precautionary and emergency handling of the material by a properly trained person having the necessary technical skills.  Users should consider this data only as a supplement to other information gathered and must make independent determinations of suitability and completeness of information from all sources to assure proper use, storage and disposal of these materials and the safety and health of employees and customers and the protection of the environment.  WE MAKE NO REPRESENTATION OR WARRANTY OF ANY KIND, EXPRESS OR IMPLIED, INCLUDING MERCHANTABILITY OR FITNESS FOR A PARTICULAR USE, WITH RESPECT TO THE INFORMATION HEREIN OR THE PRODUCT TO WHICH THE INFORMATION REFERS.