MSDS Number: T4914 * * * * * Effective Date: 07/06/06 * * * * * Supercedes: 02/16/06

# 1,1,1-TRICHLOROETHANE

## 1. Product Identification

Synonyms: Methyl chloroform; trichloroethane; chloroetene 
CAS No.: 71-55-6 
Molecular Weight: 133.40 
Chemical Formula: CH3CCl3 
Product Codes: 9435, 9437, W509, W510

## 2. Composition/Information on Ingredients

Ingredient                    CAS No         Percent        Hazardous

------------------------   ------------   ------------      ---------

Methyl Chloroform            71-55-6        96 - 100%          Yes

## 3. Hazards Identification

Emergency Overview 
-------------------------- 
WARNING! HARMFUL IF SWALLOWED, INHALED OR ABSORBED THROUGH SKIN. AFFECTS CENTRAL NERVOUS SYSTEM, LIVER, KIDNEYS, AND CARDIOVASCULAR SYSTEM. CAUSES IRRITATION TO SKIN, EYES AND RESPIRATORY TRACT.  

J.T. Baker SAF-T-DATA(tm) Ratings (Provided here for your convenience) 
----------------------------------------------------------------------------------------------------------- 
Health Rating: 2 - Moderate 
Flammability Rating: 1 - Slight 
Reactivity Rating: 1 - Slight 
Contact Rating: 2 - Moderate 
Lab Protective Equip: GOGGLES; LAB COAT; VENT HOOD; PROPER GLOVES 
Storage Color Code: Blue (Health) 
----------------------------------------------------------------------------------------------------------- 

Potential Health Effects 
---------------------------------- 

Inhalation: 
Inhalation of vapors will irritate the respiratory tract. Affects the central nervous system. Symptoms include headache, dizziness, weakness, nausea. Higher levels of exposure (> 5000 ppm) can cause irregular heart beat, kidney and liver damage, fall in blood pressure, unconsciousness and even death. 
Ingestion: 
Harmful if swallowed. Symptoms similar to inhalation will occur along with nausea, vomiting. Aspiration of material into the lungs can cause chemical pneumonitis which can be fatal. If aspirated, may be rapidly absorbed through the lungs and result in injury to other body systems. 
Skin Contact: 
Causes mild irritation and redness, especially on prolonged contact. Repeated contact may cause drying or flaking of the skin. 
Eye Contact: 
Liquids and vapors cause irritation. Symptoms include tearing, redness, stinging, swelling. 
Chronic Exposure: 
Prolonged or repeated skin contact may cause dermatitis. Chronic exposure may affect the kidneys and liver. Dioxane is a suspected human carcinogen based on animal data. 
Aggravation of Pre-existing Conditions: 
Personnel with CNS, kidney, liver or heart disease may be more susceptible to the effects of this substance. Use of alcoholic beverages may aggravate symptoms.

## 4. First Aid Measures

Inhalation: 
Remove to fresh air. If not breathing, give artificial respiration. If breathing is difficult, give oxygen. Call a physician. 
Ingestion: 
If swallowed, DO NOT INDUCE VOMITING. Give large quantities of water. Never give anything by mouth to an unconscious person. Get medical attention immediately. 
Skin Contact: 
In case of contact, immediately flush skin with plenty of soap and water for at least 15 minutes while removing contaminated clothing and shoes. Wash clothing before reuse. Call a physician. 
Eye Contact: 
Immediately flush eyes with plenty of water for at least 15 minutes, lifting lower and upper eyelids occasionally. Get medical attention immediately.

## 5. Fire Fighting Measures

Fire: 
Autoignition temperature: 500C (932F)
Flammable limits in air % by volume: 
lel: 7.0; uel: 16.0
Vapors in containers can explode if subjected to high energy source.
Dioxane has a flash point below 16C (60F). 
Explosion: 
Can react with strong caustic, such as potash to form a flammable or explosive material. Air/vapor mixtures may explode when heated. Vapors can flow along surfaces to distant ignition source and flash back. Sealed containers may rupture when heated. 
Fire Extinguishing Media: 
Use any means suitable for extinguishing surrounding fire. 
Special Information: 
In the event of a fire, wear full protective clothing and NIOSH-approved self-contained breathing apparatus with full facepiece operated in the pressure demand or other positive pressure mode. Combustion by-products include phosgene and hydrogen chloride gases. Structural firefighters' clothing provides only limited protection to the combustion products of this material.

## 6. Accidental Release Measures

Ventilate area of leak or spill. Remove all sources of ignition. Wear appropriate personal protective equipment as specified in Section 8. Isolate hazard area. Keep unnecessary and unprotected personnel from entering. Contain and recover liquid when possible. Use non-sparking tools and equipment. Collect liquid in an appropriate container or absorb with an inert material (e. g., vermiculite, dry sand, earth), and place in a chemical waste container. Do not use combustible materials, such as saw dust. Do not flush to sewer! Do not use aluminum, magnesium or zinc metal for storage container. US Regulations (CERCLA) require reporting spills and releases to soil, water and air in excess of reportable quantities. The toll free number for the US Coast Guard National Response Center is (800) 424-8802.

## 7. Handling and Storage

Keep in a tightly closed container, stored in a cool, dry, ventilated area. Protect against physical damage. Isolate from any source of heat or ignition. Containers of this material may be hazardous when empty since they retain product residues (vapors, liquid); observe all warnings and precautions listed for the product. Do not use aluminum equipment or storage containers. Contact with aluminum parts in a pressurized fluid system may cause violent reactions.

## 8. Exposure Controls/Personal Protection

Airborne Exposure Limits: 
-OSHA Permissible Exposure Limit (PEL):
350 ppm (TWA) for trichloroethane
-ACGIH Threshold Limit Value (TLV):
350 ppm (TWA), 450 ppm (STEL) for trichloroethane
Ventilation System: 
A system of local and/or general exhaust is recommended to keep employee exposures below the Airborne Exposure Limits. Local exhaust ventilation is generally preferred because it can control the emissions of the contaminant at its source, preventing dispersion of it into the general work area. Please refer to the ACGIH document, Industrial Ventilation, A Manual of Recommended Practices, most recent edition, for details. 
Personal Respirators (NIOSH Approved): 
If the exposure limit is exceeded and engineering controls are not feasible, wear a supplied air, full-facepiece respirator, airlined hood, or full-facepiece self-contained breathing apparatus. Breathing air quality must meet the requirements of the OSHA respiratory protection standard (29CFR1910.134). This substance has questionable warning properties. Where respirators are required, you must have a written program covering the basic requirements in the OSHA respirator standard. These include training, fit testing, medical approval, cleaning, maintenance, cartridge change schedules, etc. See 29CFR1910.134 for details. 
Skin Protection: 
Wear impervious protective clothing, including boots, gloves, lab coat, apron or coveralls, as appropriate, to prevent skin contact. Viton is a recommended material for personal protective equipment. 
Eye Protection: 
Use chemical safety goggles and/or a full face shield where splashing is possible. Maintain eye wash fountain and quick-drench facilities in work area.

## 9. Physical and Chemical Properties

Appearance: 
Clear, colorless liquid. 
Odor: 
Mild chloroform-like odor. 
Solubility: 
4,400 ppm in water @ 20C (68F) 
Specific Gravity: 
1.34 @ 20C/4C 
pH: 
No information found. 
% Volatiles by volume @ 21C (70F): 
100 
Boiling Point: 
74C (165F) 
Melting Point: 
-32C (-26F) 
Vapor Density (Air=1): 
4.63 
Vapor Pressure (mm Hg): 
100 @ 20C (68F) 
Evaporation Rate (BuAc=1): 
12.8

## 10. Stability and Reactivity

Stability: 
Requires inhibitor content to prevent corrosion of metals. Slowly hydrolyzes in water to form hydrochloric and acetic acid. 
Hazardous Decomposition Products: 
May produce carbon monoxide, carbon dioxide, hydrogen chloride and phosgene when heated to decomposition. Carbon dioxide and carbon monoxide may form when heated to decomposition. 
Hazardous Polymerization: 
Hazardous polymerization can occur in contact with aluminum trichloride. 
Incompatibilities: 
Open flames, welding arcs, nitrogen tetroxide, oxygen, liquid oxygen, sodium, sodium hydroxide, and sodium-potassium alloy, strong alkalis, oxidizers, aluminum and other reactive metals. 
Conditions to Avoid: 
Insufficient inhibitor, incompatibles, heat, flame and ignition sources

## 11. Toxicological Information

Oral rat LD50: 9600 mg/kg; inhalation rat LC50: 18000 ppm/4H; investigated as a mutagen, tumorigen, reproductive effector; irritation eye rabbit, Standard Draize, 2mg/24H severe.

-\Cancer Lists\------------------------------------------------------

---NTP Carcinogen---

Ingredient                       Known    Anticipated    IARC Category

------------------------------   -----    -----------    -------------

Methyl Chloroform (71-55-6)        No          No             3

## 12. Ecological Information

Environmental Fate: 
When released into the soil, this material is not expected to biodegrade. When released into the soil, this material is expected to leach into groundwater. When released into the soil, this material is expected to quickly evaporate. When released to water, this material is expected to quickly evaporate. This material is not expected to significantly bioaccumulate. When released into the air, this material may be removed from the atmosphere to a moderate extent by wet deposition. When released to the atmosphere, this material has an average global half-life of 6.0 - 6.9 years. When released into the air, this material may adversely affect the ozone layer. 
Environmental Toxicity: 
This material is expected to be slightly toxic to aquatic life. The LC50/96-hour values for fish are between 10 and 100 mg/l.

## 13. Disposal Considerations

Whatever cannot be saved for recovery or recycling should be handled as hazardous waste and sent to a RCRA approved incinerator or disposed in a RCRA approved waste facility. Processing, use or contamination of this product may change the waste management options. State and local disposal regulations may differ from federal disposal regulations. Dispose of container and unused contents in accordance with federal, state and local requirements.

## 14. Transport Information

Domestic (Land, D.O.T.) 
----------------------- 
Proper Shipping Name: 1,1,1-TRICHLOROETHANE 
Hazard Class: 6.1 
UN/NA: UN2831 
Packing Group: III 
Information reported for product/size: 20L

## 15. Regulatory Information

-\Chemical Inventory Status - Part 1\---------------------------------

Ingredient                                 TSCA  EC   Japan  Australia

-----------------------------------------  ----  ---  -----  ---------

Methyl Chloroform (71-55-6)                 Yes  Yes   Yes      Yes

-\Chemical Inventory Status - Part 2\---------------------------------

--Canada--

Ingredient                                   Korea  DSL   NDSL  Phil.

-------------------------------------------  -----  ---   ----  -----

Methyl Chloroform (71-55-6)                   Yes   Yes   No     Yes

-\Federal, State & International Regulations - Part 1\----------------

-SARA 302-    ------SARA 313------

Ingredient                           RQ    TPQ     List  Chemical Catg.

----------------------------------   ---   -----   ----  --------------

Methyl Chloroform (71-55-6)          No    No      Yes        No

-\Federal, State & International Regulations - Part 2\----------------

-RCRA-    -TSCA-

Ingredient                                 CERCLA     261.33     8(d)

---------------------------------------    ------     ------    ------

Methyl Chloroform (71-55-6)                1000       U226       No

Chemical Weapons Convention:  No     TSCA 12(b):  No     CDTA:  No

SARA 311/312:  Acute: Yes      Chronic: Yes  Fire: No  Pressure: No

Reactivity: No          (Mixture / Liquid)

WARNING: 
THIS PRODUCT CONTAINS A CHEMICAL(S) KNOWN TO THE STATE OF CALIFORNIA TO CAUSE CANCER. 

Australian Hazchem Code: 2[Z] 
Poison Schedule: S6 
WHMIS: 
This MSDS has been prepared according to the hazard criteria of the Controlled Products Regulations (CPR) and the MSDS contains all of the information required by the CPR.

## 16. Other Information

NFPA Ratings: Health: 2 Flammability: 1 Reactivity: 0 
Label Hazard Warning: 
WARNING! HARMFUL IF SWALLOWED, INHALED OR ABSORBED THROUGH SKIN. AFFECTS CENTRAL NERVOUS SYSTEM, LIVER, KIDNEYS, AND CARDIOVASCULAR SYSTEM. CAUSES IRRITATION TO SKIN, EYES AND RESPIRATORY TRACT.  
Label Precautions: 
Avoid breathing vapor.
Keep container closed.
Use only with adequate ventilation.
Wash thoroughly after handling.
Avoid contact with eyes, skin and clothing. 
Label First Aid: 
If swallowed, DO NOT INDUCE VOMITING. Give large quantities of water. Never give anything by mouth to an unconscious person. If inhaled, remove to fresh air. If not breathing, give artificial respiration. If breathing is difficult, give oxygen. In case of contact, immediately flush eyes or skin with plenty of water for at least 15 minutes while removing contaminated clothing and shoes. Wash clothing before reuse. In all cases call a physician. 
Product Use: 
Laboratory Reagent. 
Revision Information: 
No Changes. 
Disclaimer: 
************************************************************************ 
Mallinckrodt Baker, Inc. provides the information contained herein in good faith but makes no representation as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose. MALLINCKRODT BAKER, INC. MAKES NO REPRESENTATIONS OR WARRANTIES, EITHER EXPRESS OR IMPLIED, INCLUDING WITHOUT LIMITATION ANY WARRANTIES OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE WITH RESPECT TO THE INFORMATION SET FORTH HEREIN OR THE PRODUCT TO WHICH THE INFORMATION REFERS. ACCORDINGLY, MALLINCKRODT BAKER, INC. WILL NOT BE RESPONSIBLE FOR DAMAGES RESULTING FROM USE OF OR RELIANCE UPON THIS INFORMATION. 
************************************************************************ 
Prepared by: Environmental Health & Safety
Phone Number: (314) 654-1600 (U.S.A.)