Material Safety Data Sheet

Revision Number: 1

Issue date: 7/26/2009

1. PRODUCT AND COMPANY IDENTIFICATION

Product name:   Magnesium Touch-Up Kit

Product type: Conversion coating

Company address:

DGR Industrial Products, Inc.

4435 North First Street, Suite #184

Livermore, California 94551

Phone:  (408) 221-7122

For Chemical Emergency: Call CHEMTREC at 800.424.9300

Internet: www.chemical-supermarket.com

2. HAZARDS IDENTIFICATION

EMERGENCY OVERVIEW

HMIS:

Physical state: Liquid 					HEALTH: 		3

Color: Orange 						FLAMMABILITY: 	0

Odor: None 						PHYSICAL HAZARD: 	0

Personal Protection: See MSDS Section 8

DANGER—CORROSIVE!: CAUSES EYE, SKIN AND RESPIRATORY TRACT IRRITATION.

MAY CAUSE EYE AND SKIN BURNS.

MAY CAUSE ALLERGIC SKIN REACTION.

POSSIBLE CANCER HAZARD.

Relevant routes of exposure: Skin, Inhalation, Eyes

Potential Health Effects

Inhalation: Mists, vapors or liquid may cause severe irritation or burns. Prolonged or repeated breathing

may cause ulceration of nasal membranes.

Skin contact: Contact with broken skin may lead to formation of firmly marginated "chrome sores". Product contains chromium, which may cause an allergic skin sensitization reaction. Massive

overexposures may lead to kidney failure and death. A component in this product may be

harmful or fatal if absorbed through the skin, especially if skin is damaged.

Eye contact: This product is severely irritating to the eyes and may cause irreversible damage including

burns and blindness. Contact with this product may cause severe eye damage.

Ingestion: This product may produce corrosive damage to the gastrointestinal tract if it is swallowed.

Existing conditions aggravated by exposure:

Eye, skin and respiratory disorders. Kidney disorders.

This material is considered hazardous by the OSHA Hazard Communication Standard (29 CFR

1910.1200).

See Section 11 for additional toxicological information.

3. COMPOSITION / INFORMATION ON INGREDIENTS

Hazardous components 		CAS NUMBER 			%

Chromic acid 			7738-94-5 			1 – 5

4. FIRST AID MEASURES

Inhalation: If mist or vapor of this product is inhaled, remove person immediately to fresh

air. Seek medical attention if symptoms develop or persist.

Skin contact: Remove contaminated clothing and footwear. For skin contact, flush with large

amounts of water. Seek immediate medical attention. If irritation persists,

repeat flushing and get medical attention. Discard any shoes or clothing items

that cannot be decontaminated.

Eye contact: In case of contact with the eyes, rinse immediately with plenty of water for 15

minutes, and seek immediate medical attention.

Ingestion: Get immediate medical attention. Do not induce vomiting. Give one to two

glasses of water or milk. Never give anything by mouth to a victim who is

unconscious or is having convulsions.

Notes to physician: Treat symptomatically and supportively.

5. FIRE FIGHTING MEASURES

Flash point: 121.1 °C (249.98 °F) calculated

Autoignition temperature: Not applicable

Flammable/Explosive limits - lower: Not applicable

Flammable/Explosive limits - upper: Not applicable

Extinguishing media: Use media appropriate for surrounding material.

Special firefighting procedures: Wear full protective clothing. Wear self-contained breathing apparatus.

Unusual fire or explosion hazards: This product is an aqueous mixture which will not burn.

Hazardous combustion products: Irritating and toxic gases or fumes may be released during a fire.

6. ACCIDENTAL RELEASE MEASURES

Use personal protection recommended in Section 8, isolate the hazard area and deny entry to unnecessary and unprotected personnel.

Environmental precautions: Prevent further leakage or spillage if safe to do so. Wear appropriate

protective equipment and clothing during clean-up. Block any potential routes

to water systems.

Clean-up methods: Absorb spill with inert material. Shovel material into appropriate container for

disposal.

7. HANDLING AND STORAGE

Handling: Prevent contact with eyes, skin and clothing. Do not breathe vapor and mist.

Wash thoroughly after handling. Provide adequate ventilation. For industrial

use only. Do not take internally.

Storage: For safe storage, store between 4.4 °C (39.9 °F) and 37.8 °C (100°F)

Keep container tightly closed and in a cool, well-ventilated place away from incompatible materials. Protect from freezing.  For information on product shelf life, please review labels on container or check the Technical Process Instruction.

8. EXPOSURE CONTROLS / PERSONAL PROTECTION

Employers should complete an assessment of all workplaces to determine the need for, and selection of, proper exposure controls and protective equipment for each task performed.

Hazardous components 	ACGIH 	TLV 		OSHA PEL 	AIHA WEEL 	OTHER

Chromic acid 		0.05 mg/m3 TWA				       None		  None

(as Cr)			0.1 mg/m3 Ceiling

0.005 mg/m3 TWA

(SKIN)0.0025 mg/m3

OSHA ACT

29 CFR 1910.1026

Engineering controls: Provide local and general exhaust ventilation to effectively remove and

prevent buildup of any vapors or mists generated from the handling of this product.

Respiratory protection: If ventilation is not sufficient to effectively prevent buildup of aerosols, mists or

vapors, appropriate NIOSH/MSHA respiratory protection must be provided.

Eye/face protection: Wear chemical goggles; face shield (if splashing is possible).

Skin protection: Chemical resistant, impermeable gloves.  Gloves should be tested to determine suitability for prolonged contact.  Use of impervious apron and boots are recommended.

9. PHYSICAL AND CHEMICAL PROPERTIES

Physical state:	 			Liquid

Color: 					Orange

Odor: 					None

Odor threshold: 				Not available

pH: 					1.2 - 1.4

Vapor pressure: 				Not determined

Boiling point/range: 			200 °F (93.3 °C)

Melting point/ range: 			Not available

Specific gravity: 				1.00 - 1.03 at 15.6 °C (60.08 °F)

Vapor density: 				Not determined

Flash point: 				121.1 °C (249.98 °F) calculated

Flammable/Explosive limits - lower: 	Not applicable

Flammable/Explosive limits - upper: 	Not applicable

Autoignition temperature: 			Not applicable

Evaporation rate: 				Not available

Solubility in water: 			Complete

Partition coefficient (n-octanol/water): 	Not determined

VOC content: 				Not applicable

10. STABILITY AND REACTIVITY

Stability: Stable at normal conditions.

Hazardous reactions: Will not occur.

Hazardous decomposition products: Irritating and/or toxic fumes and gases may be emitted upon the product's decomposition.

Incompatible materials: Avoid contact with organic materials, oils, greases, and any oxidizable

materials.

Conditions to avoid: None identified.

11. TOXICOLOGICAL INFORMATION

Hazardous components:		Chromic Acid

NTP Carcinogen:			Known carcinogen

IARC Carcinogen:		Group 1

OSHA Carcinogen:		Yes

Hazardous components 			Health Effects/Target Organs

Chromic acid				Allergen, Blood, Human carcinogen,

Central nervous system, Corrosive,

Developmental, Eyes, Gastrointestinal tract,

Irritant, Kidney, Liver, Mutagen,

Reproductive, Respiratory

12. ECOLOGICAL INFORMATION

Ecological information: Not available

13. DISPOSAL CONSIDERATIONS

Information provided is for unused product only.  Recommended method of disposal: Follow all local, state, federal and provincial regulations for disposal.

Hazardous waste number:  This product, if discarded directly, would be a characteristic RCRA corrosive

waste (D002).  This product contains chromium which is a hazardous waste (D007).

14. TRANSPORT INFORMATION

U.S. Department of Transportation Ground (49 CFR)

Proper shipping name: Chromic acid solution

Hazard class or division: 8

Identification number: UN 1755

Packing group: III

International Air Transportation (ICAO/IATA)

Proper shipping name: Chromic acid solution

Hazard class or division: 8

Identification number: UN 1755

Packing group: III

Water Transportation (IMO/IMDG)

Proper shipping name: CHROMIC ACID SOLUTION

Hazard class or division: 8

Identification number: UN 1755

Packing group: III

15. REGULATORY INFORMATION

United States Regulatory Information

TSCA 8 (b) Inventory Status: All components are listed or are exempt from listing on the Toxic Substances Control Act Inventory.

TSCA 12(b) Export Notification: Chromic acid (CAS# 7738-94-5).

CERCLA/SARA Section 302 EHS: None above reporting de minimus

CERCLA/SARA Section 311/312: Immediate Health, Delayed Health

CERCLA/SARA 313: This product contains the following toxic chemicals subject to the reporting requirements of section 313 of the Emergency Planning and Community Right-To-Know Act of 1986 (40

CFR 372). Chromic acid (CAS# 7738-94-5).

CERCLA Reportable quantity: Chromic acid (CAS# 7738-94-5) 10 lbs. (4.54 kg)

California Proposition 65: This product contains a chemical known in the State of California to cause cancer.

Canada Regulatory Information

CEPA DSL/NDSL Status: All components are listed on or are exempt from listing on the Canadian Domestic Substances List.

WHMIS hazard class: D.2.A, D.2.B, E

16. OTHER INFORMATION

DISCLAIMER: The data contained in this document is furnished for information only and is believed to be reliable.  However, DGR Industrial Products, Inc. does not assume responsibility for any results obtained by persons over whose methods DGR Industrial Products, Inc. has no control.  It is the user's responsibility to determine the suitability of DGR’s products or any production methods mentioned herein for a particular purpose, and to adopt such precautions as may be advisable for the protection of property and persons against any hazards that may be involved in the handling and use of any of DGR’s products.  In light of the foregoing, DGR Industrial Products, Inc. specifically disclaims all warranties, express or implied, including warranties of merchantability and fitness for a particular purpose, arising from sale or use of DGR’s products.  DGR Industrial Products, Inc. further disclaims any liability for consequential or incidental damages of any kind, including lost profits.