Effective Date:  2/24/08

# “Super Alloy” Surface Etchant

## 1. Product Identification

Product Name:

Super Alloy Etching Solution, SKU #1203-136

Purpose:

An aqueous solution of aggressive industrial acids formulated specifically for surface etching of the “Super Alloys” including nickel, cobalt, and nickel/iron alloys to facilitate metallographic inspection.

Manufacturer:

DGR Industrial Products, Inc.

4435 North First Street, Suite #184

Livermore, Ca 94551

(408) 221-7122

www.chemical-supermarket.com

## 2. Composition/Information on Ingredients

Ingredient                     CAS No      Percent (v/v)     Hazardous

-------------------------   ------------   ------------      ---------

Hydrochloric Acid (38%)      7647-01-0         30-45%          Yes

Ethyl Alcohol		      64-17-5           30-45%          Yes

Copper (II) Chloride         7447-39-4          5-10%          Yes

## 3. Hazards Identification—Emergency Overview

POISON! DANGER! CORROSIVE. LIQUID AND MIST CAUSE SEVERE BURNS TO ALL BODY TISSUE. MAY BE FATAL IF SWALLOWED OR INHALED. INHALATION MAY CAUSE LUNG DAMAGE. 

SAF-T-DATA(tm) Ratings (Provided here for your convenience) 
----------------------------------------------------------------------------------------------------------- 
Health Rating: 3 - Severe 
Flammability Rating: 3 - Severe 
Reactivity Rating: 1 - Slight 
Contact Rating: 3 – Severe (Corrosive) 
Lab Protective Equip: GOGGLES & SHIELD; LAB COAT & APRON; VENT HOOD; PROPER GLOVES 
Storage Color Code: White (Corrosive) 
----------------------------------------------------------------------------------------------------------- 

Potential Health Effects 
---------------------------------- 

Inhalation: 
Inhalation produces damaging effects on the mucous membranes and upper respiratory tract. Symptoms may include irritation of the nose and throat, and labored breathing. May cause lung edema, a medical emergency. 
Ingestion: 
Corrosive. Swallowing can cause severe burns of the mouth, throat, and stomach, leading to death. Can cause sore throat, vomiting, diarrhea. Circulatory collapse with clammy skin, weak and rapid pulse, shallow respirations, and scanty urine may follow ingestion or skin contact. Circulatory shock is often the immediate cause of death. 
Skin Contact: 
Corrosive. Symptoms of redness, pain, and severe burn can occur. Circulatory collapse with clammy skin, weak and rapid pulse, shallow respirations, and scanty urine may follow skin contact or ingestion. Circulatory shock is often the immediate cause of death. 
Eye Contact: 
Corrosive. Contact can cause blurred vision, redness, pain and severe tissue burns. Can cause blindness. 
Chronic Exposure: 
Long-term exposure to mist or vapors may cause damage to teeth. Chronic exposure to mists containing sulfuric acid is a cancer hazard. 
Aggravation of Pre-existing Conditions: 
Persons with pre-existing skin disorders or eye problems or impaired respiratory function may be more susceptible to the effects of the substance.

## 4. First Aid Measures

Inhalation: 
Remove to fresh air. If not breathing, give artificial respiration. If breathing is difficult, give oxygen. Call a physician immediately. 
Ingestion: 
DO NOT INDUCE VOMITING. Give large quantities of water. Never give anything by mouth to an unconscious person. Call a physician immediately. 
Skin Contact: 
In case of contact, immediately flush skin with plenty of water for at least 15 minutes while removing contaminated clothing and shoes. Wash clothing before reuse. Excess acid on skin can be neutralized with a 2% solution of bicarbonate of soda. Call a physician immediately.

Eye Contact: 
Immediately flush eyes with gentle but large stream of water for at least 15 minutes, lifting lower and upper eyelids occasionally. Call a physician immediately.

## 5. Fire Fighting Measures

Fire: 
Concentrated material is a strong dehydrating agent. Reacts with organic materials and may cause ignition of finely divided materials on contact. 
Explosion: 
Contact with most metals can cause formation of flammable and explosive hydrogen gas. 
Fire Extinguishing Media: 
Dry chemical, foam or carbon dioxide. Water spray may be used to keep fire exposed containers cool. 
Special Information: 
In the event of a fire, wear full protective clothing and NIOSH-approved self-contained breathing apparatus with full facepiece operated in the pressure demand or other positive pressure mode. Structural firefighter's protective clothing is ineffective for fires involving this material. Stay away from sealed containers.

## 6. Accidental Release Measures

Ventilate area of leak or spill. Wear appropriate personal protective equipment as specified in Section 8.  Isolate hazard area.  Keep unnecessary and unprotected personnel from entering.  Contain and recover liquid when possible.  Neutralize with alkaline material (soda ash, lime), then absorb with an inert material (e. g., vermiculite, dry sand, earth), and place in a chemical waste container.  Do not use combustible materials, such as saw dust.

## 7. Handling and Storage

Store in a cool, dry, ventilated storage area with acid resistant floors and good drainage. Protect from physical damage.  Keep out of direct sunlight and away from heat, water, and incompatible materials.  Do not wash out container and use it for other purposes. When diluting, always add the acid to water; never add water to the acid.  Containers of this material may be hazardous when empty since they retain product residues (vapors, liquid); observe all warnings and precautions listed for the product.

## 8. Exposure Controls/Personal Protection

Ventilation System: 
A system of local and/or general exhaust is recommended to keep employee exposures below the Airborne Exposure Limits. Local exhaust ventilation is generally preferred because it can control the emissions of the contaminant at its source, preventing dispersion of it into the general work area.

Skin Protection: 
Wear impervious protective clothing, including boots, gloves, lab coat, apron or coveralls, as appropriate, to prevent skin contact.

Eye Protection: 
Use chemical safety goggles and/or a full face shield where splashing is possible. Maintain eye wash fountain and quick-drench facilities in work area.

## 9. Physical and Chemical Properties

Appearance: 			Clear liquid. 
Odor: 				Odorless. 
Solubility: 			Miscible with water 
Specific Gravity: 		1.07 (10%) 
pH: 				2 
% Volatiles by volume @ 21C (70F): ``No information found.

## 10. Stability and Reactivity

Stability: 
Stable under ordinary conditions of use and storage. 
Hazardous Decomposition Products: 
Toxic fumes of oxides of sulfur when heated to decomposition.  Will react with water or steam to produce toxic and corrosive fumes.  Reacts with carbonates to generate carbon dioxide gas, and with cyanides and sulfides to form poisonous hydrogen cyanide and hydrogen sulfide respectively.

Hazardous Polymerization: 
Will not occur. 
Incompatibilities: 
Hydrochloric acid is highly reactive with strong bases, metals, metal oxides, hydroxides, amines, carbonates and other alkaline materials. Incompatible with materials such as cyanides, sulfides, sulfites, and formaldehyde.

Conditions to Avoid: 
Heat, incompatibles.

## 11. Toxicological Information

Toxicological Data: 
Oral rat LD50: 2140 mg/kg; inhalation rat LC50: 510 mg/m3/2H; standard Draize, eye rabbit, 250 ug (severe); investigated as a tumorigen, mutagen, reproductive effector. 
Carcinogenicity: 
Cancer Status: The International Agency for Research on Cancer (IARC) has classified "strong inorganic acid mists containing sulfuric acid" as a known human carcinogen, (IARC category 1). This classification applies only to mists containing sulfuric acid and not to sulfuric acid or sulfuric acid solutions.

-\Cancer Lists\------------------------------------------------------

---NTP Carcinogen---

Ingredient                      Known    Anticipated    IARC Category

-----------------------------   -----    -----------    -------------

Hydrogen Chloride (7647-01-0)	   No		No		3

Ethyl Alcohol (64-17-5)	   No		No		3

Copper(II)Chloride(7447-39-4)	   No		No		3

## 12. Ecological Information

Environmental Fate: 
When released into the soil, this material may leach into groundwater. When released into the air, this material may be removed from the atmosphere to a moderate extent by wet deposition. When released into the air, this material may be removed from the atmosphere to a moderate extent by dry deposition. 
Environmental Toxicity: 
LC50 Flounder 100 to 330 mg/l/48 hr aerated water/Conditions of bioassay not specified; LC50 Shrimp 80 to 90 mg/l/48 hr aerated water /Conditions of bioassay not specified; LC50 Prawn 42.5 ppm/48 hr salt water /Conditions of bioassay not specified.
This material may be toxic to aquatic life.

## 13. Disposal Considerations

Whatever cannot be saved for recovery or recycling should be handled as hazardous waste and sent to a RCRA approved incinerator or disposed in a RCRA approved waste facility.  Processing, use or contamination of this product may change the waste management options.  State and local disposal regulations may differ from federal disposal regulations.  Dispose of container and unused contents in accordance with federal, state and local requirements.

## 14. Transport Information

Domestic (Land, D.O.T.) 
----------------------- 
Proper Shipping Name: Acidic Etching Solution 
Hazard Class: 8 
UN/NA: 	n/a 
Packing Group: III

## 15. Regulatory Information

Poison Schedule: None allocated.

## 16. Other Information

NFPA Ratings: Health: 3 Flammability: 0 Reactivity: 2
Label Hazard Warning: 
POISON! DANGER! CORROSIVE. LIQUID AND MIST CAN CAUSE SEVERE BURNS TO ALL BODY TISSUE. MAY BE FATAL IF SWALLOWED OR CONTACTED WITH SKIN. HARMFUL IF INHALED.

Label Precautions: 
Do not get in eyes, on skin, or on clothing.
Do not breathe mist.
Keep container closed.
Use only with adequate ventilation.
Wash thoroughly after handling.
Do not contact with water.

Label First Aid: 
In all cases call a physician immediately. In case of contact, immediately flush eyes or skin with plenty of water for at least 15 minutes while removing contaminated clothing and shoes. Wash clothing before re-use. Excess acid on skin can be neutralized with a 2% bicarbonate of soda solution. If swallowed, DO NOT INDUCE VOMITING. Give large quantities of water. Never give anything by mouth to an unconscious person. If inhaled, remove to fresh air. If not breathing, give artificial respiration. If breathing is difficult, give oxygen.

Product Use: 
Industrial use only. 
Revision Information: 
MSDS Section(s) changed since last revision of document include: 8. 
Disclaimer: 
************************************************************************ 
The information contained herein is provided in good faith but with no representation as to its comprehensiveness or accuracy.  This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product.  Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

WE MAKE NO REPRESENTATIONS OR WARRANTIES, EITHER EXPRESS OR IMPLIED, INCLUDING WITHOUT LIMITATION ANY WARRANTIES OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE WITH RESPECT TO THE INFORMATION SET FORTH HEREIN OR THE PRODUCT TO WHICH THE INFORMATION REFERS.  ACCORDINGLY, WE WILL NOT BE RESPONSIBLE FOR DAMAGES RESULTING FROM USE OF OR RELIANCE UPON THIS INFORMATION. 
************************************************************************